Lab 22 HW exercise

First - Make rotated sesimograms

• read CCM horizontals IU.CCM..BH[EN].M.1991.292.213645.SAC (they are in the updated sacdata directory)

• Remove the trend

• Rotate the seismograms

• Remove the trend again (Probably too many rtrends, but they don’t do any harm)

• Look at interesting part – xlim command - surface waves go from 2200 3700

• Save the rotated data – write to disk – use the azimuth to identify them, the radial component is first (189°, can check against azimuth in header), and the transverse component is second (put in the appropriate value for XXX)

w sac IU.CCM..BH189.M.1991.292.213645.SAC IU.CCM..BHXXX.M.1991.292.213645.SAC

Rotated seismograms now saved to disk and still in memory

Second – get Hilbert transform the radial component

• cut to interesting part

• Read in radial component (this erases everything else)

• Remove the trend

• Take the Hilbert transform

• Remove the trend

• Save the Hilbert transform of the radial component

w sac IU.CCM..BH189Hilbert.M.1991.292.213645.SAC

Third - Plot Hilbert transform of radial and the  vertical on same plot

• Read in the vertical component

• color on increment on list black red  <<< turn on color, get increments from color list

• Remove the means

• Plot everything on a single plot.

What you should see. What are you looking at?